File E132002

Project 05CA47626

November 10, 2005

REPORT

On

COMPONENT - POWER SUPPLIES, INFORMATION TECHNOLOGY EQUIPMENT

Astec International Ltd.

Kowloon, Hong Kong

Copyright © 2005 Underwriters Laboratories Inc.

Underwriters Laboratories Inc. authorizes the above-named company to

reproduce this Report provided it is reproduced in its entirety.

Underwriters Laboratories Inc. authorizes the above-named company to reproduce the latest pages of that portion of this Report consisting of this Cover Page through Page 3.

File E132002	Vol. 13	Sec. 3	Page 1	Issued:  2005-11-10

and Report	Revised:  2010-06-03

# DESCRIPTION

# PRODUCT COVERED

*USR, CNR Component – DC-DC converters, Models AVO100-48S3V3,

AVO75-48S12, AVO75-48S2V5, AVO50-48S12,       AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2,  ALO30F48N, AVO75B-48S3V3, AVO50B-48S3V3 and AVO100B-48S3V3 for use in Information Technology Equipment.

# ELECTRICAL RATINGS:

| MODEL | INPUT | OUTPUT |
| --- | --- | --- |
| AVO100-48S3V3 | DC +36-+75 V | +3.3V, 30A |
| or | 3.5A |  |
| ALO30F48N |  |  |
| AVO75-48S12 | DC +36-+75 V,3.0A | +12V, 6.3A |
| AVO75-48S2V5 | DC +36-+75 V,2.5A | +2.5V, 25A |
| AVO50-48S12 | DC +36-+75 V,2.0A | +12V, 4.2A |
| AVO50-48S2V5 | DC +36-+75 V,2.0A | +2.5V, 20A |
| AVO75-48S05 | DC +36-+75 V,2.3A | +5V, 15A |
| AVO50-48S05 | DC +36-+75 V,1.6A | +5V, 10A |
| AVO75-48S1V8 | DC +36-+75 V,2.0A | +1.8V, 25A |
| AVO75-48S1V2 | DC +36-+75 V,1.5A | +1.2V, 25A |
| AVO50-48S1V8 | DC +36-+75 V,1.5A | +1.8V, 20A |
| AVO50-48S1V2 | DC +36-+75 V,1.0A | +1.2V, 20A |
| AVO75B-48S3V3 | DC +36-+75 V,2.5A | +3.3V, 20A |
| AVO50B-48S3V3 | DC +36-+75 V,2.0A | +3.3V, 15A |
| AVO100B-48S3V3 | DC +36-+75 V,3.5A | +3.3V, 30A |

Remark:

For EMERSON series models, AVO100-48S3V3, AVO75-48S12, AVO75-48S2V5, AVO50-48S12, AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8,        AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2, AVO75B-48S3V3 and AVO50B-48S3V3 series, the model name may contain suffix,  "P-L" and/or SRWXYZ, where P: may be Null, or "P", which indicates the CNT Negative logic or Positive logic; the "-" may be Null; where L: may be Null, "2", "4", "6", "7" or "8", which indicates different length of pins; where S: may be “S” or Null, which indicates SMT encapsulation; where R: may be Null, ”L” or ”Y”: which indicates RoHS state; where W, X, Y, Z may be represented by any ASCII character code, which only relate to the different name designation.

# *

File E132002	Vol. 13	Sec. 3	Page 1A	Issued:  2005-11-10

and Report	Revised:  2010-06-03

For EMERSON model AVO100B-48S3V3 series, the model name may contain suffix, "P-B-L" and/or SRWXYZ, where P: may be Null, or "P", which indicates the CNT Negative logic or Positive logic; where B: may be "B", or Null, indicates the model has a heat sink case or not; the "-" may be Null; where L: may be Null, "2", "4", "6", "7" or "8", which indicates different length of pins; where S: may be “S” or Null, which indicates SMT encapsulation; where R: may be Null, ”L” or ”Y”: which indicates RoHS state; where W, X, Y, Z may be represented by any ASCII character code, which only relate to the different name designation.

For ASTEC series models, ALO30F48 series, the model name may contain suffix, "N-L" and/or XYZ, where "N" may be "N" or Null, which indicates the CNT Negative logic or Positive logic; "-" may be Null; L: may be Null, "2", "4", "6", "7" or "8": which indicates different length of pins; X, Y, Z may be represented by any ASCII character code, which only relate to the different name designation.

# Model name corresponding relationship is showing as below.

| Emerson series models | ASTEC series models |
| --- | --- |
| AVO100-48S3V3 | ALO30F48N |
| AVO100-48S3V3P AVO75-48S12 AVO75-48S2V5 AVO50-48S12 AVO50-48S2V5 AVO75-48S05 AVO50-48S05 AVO75-48S1V8 AVO75-48S1V2 AVO50-48S1V8 # AVO50-48S1V2 AVO75B-48S3V3 AVO50B-48S3V3 AVO100B-48S3V3 | ALO30F48 --- --- --- --- --- --- --- --- --- --- --- --- --- |

File E132002	Vol. 13	Sec. 3	Page 2	Issued:  2005-11-10

and Report	Revised:  2012-12-07

# TECHNICAL CONSIDERATIONS (NOT FOR FIELD REPRESENTATIVE'S USE):

General – The units are for use in products where the acceptability of the combination is determined by Underwriters Laboratories Inc.

*Both USR and CNR indicate investigation to the Standard for Safety of Information Technology Equipment, UL 60950-1, Second Edition, and CAN/CSA-C22.2 No. 60950-1-07, with revision date 2011-12-19.

Conditions of Acceptability – When installed in the end-use equipment, the following are the consideration to be made:

1.	These DC-DC converters have been judged on the basis of the required creepages and clearances in the Second Edition of the Standard for Safety of Information Technology Equipment, UL60950-1, Second Edition CAN/CSA-C22.2 No. 60950-1-07, Sub-clause 2.10, which covers the end-use product of which the component was designed.  The functional insulations have been evaluated by conducting Component Failure Test per sub-clause 5.3.4. (c) of UL 60950-1 Second Edition, CAN/CSA-C22.2 No. 60950-1-07.

2.	These DC-DC converters have only been evaluated for use in pollution degree 2 environment.

3.	These DC-DC converters can be connected to a SELV or TNV-2 voltage DC input circuit.  Basic insulation is provided between input and output.  The output is considered as SELV.

4.	The following end-product enclosures are required: Mechanical, Fire, Electrical.  A suitable enclosure shall be provided by end-use equipment.

5.	These DC-DC converters have been evaluated for use in 25°C and 55°C ambient, except for model AVO75B-48S3V3 and AVO50B-48S3V3 have been evaluated for use in 55°C, 70°C and 85°C, and for model AVO100B-48S3V3 have been evaluated for use in 70°C.

6.	These DC-DC converters terminals and/or connectors are not investigated for field wiring.

7.	These DC-DC converters are not evaluated for end system mounting.

8.	An external recognized fuse, (JDYX2), Cooper Industries Inc Bussmann Div (E19180), Type GDA-V, rated 6.3 A, 250 V ac, was employed at the input positive line during testing for models AVO100-48S3V3 and AVO100B-48S3V3.  Additional consideration shall be given during the end product investigation for with other current limiting devices.

9.	These DC-DC converters are not intended to be repaired by service personnel in case of failure or component defect (unit can be thrown away).

File E132002	Vol. 13	Sec. 3	Page 2A	Issued:  2005-11-10

and Report	New:  2008-01-11

10.	An external recognized fuse, (JDYX2), Littelfuse Inc., Type

216005, rated 5.0 A, 250 V ac, fast acting fuse, was employed at the input positive line during testing for model AVO75-48S12, AVO75-48S2V5,   AVO50-48S12, AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2, AVO75B-48S3V3 and                AVO50B-48S3V3.

File E132002	Vol. 13	Sec. 3	Page 3	Issued:  2005-11-10

and Report	Revised:  2010-06-03

11.	These DC-DC converters are classified as level 3 as defined by UL60950–1 and CAN/CSA-C22.2 No. 60950-1-07.

12.	Heating Test was performed with forced air cooling of 1.0 m/s for Models AVO100-48S3V3, AVO75-48S12, AVO75-48S2V5, AVO50-48S12, AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2 and ALO30F48N, and 1.5 m/s for models AVO75B-48S3V3 and AVO50B-48S3V3, and 3.0m/s for model AVO100B-48S3V3.

13.	Further electrical insulation for user accessible parts should be considered in end system.

14.	Heating Test for model AVO75B-48S3V3 was performed with forced air cooling of 1.5 m/s .  The maximum ambient is 55 °C when load is 20 A; The maximum ambient is 70 °C when load is 15 A;  The maximum ambient is 85 °C when load is 10 A.

15.	Heating Test for model AVO50B-48S3V3 was performed with forced air cooling of 1.5 m/s .  The maximum ambient is 55 °C when load is 15 A; The maximum ambient is 70 °C when load is 13.5 A;  The maximum ambient is 85 °C when load is 10 A.

*16.	Model AVO75B-48S3V3, AVO50B-48S3V3 and AVO100B-48S3V3 have been evaluated for altitude up to 3000 meters above sea level.

17.   For model AVO100B-48S3V3 with heat sink, the heat sink is considered as floating or dead metal.

File E132002	Vol. 13	Sec. 3	Page 4	Issued:  2005-11-10

and Report	Revised:  2010-06-03

# CONSTRUCTION DETAILS

Spacing - the following spacings are provided in the DC-DC converter, Models AVO100-48S3V3, AVO75-48S2V5, AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2 and ALO30F48N.

1.	Minimum 1.4 mm creepages distance and minimum 1.0 mm clearance distance between input circuits and output circuits.

Spacing - the following spacings are provided in the DC-DC converter, Models AVO75-48S12 and AVO50-48S12.

2.	Minimum 1.5 mm creepages distance and minimum 1.0 mm clearance distance between input circuits and output circuits.

*Spacing - the following spacings are provided in the DC-DC converter, Models AVO75B-48S3V3, AVO50B-48S3V3 and AVO100B-48S3V3.

3.	Minimum 1.4 mm creepages distance and minimum 1.2 mm clearance distance between input circuits and output circuits.

Section General - The following construction items are described in the Section General.

| Abbreviations | C-UL Requirements |
| --- | --- |
| Factory Location and Identification | Corrosion Protection |
| Internal Wiring | Segregation |
| Wire Positioning Devices | Marking Methods |
| Markings | Internal Polymeric Materials |
| Printed Wiring Board | Wire Connections |
| Connectors and Receptacles | Capacitors |
| Optocouplers | Voltage Surge Suppressors |
| Insulating Tubing/sleeving | Earthing/Bonding |
| Earthing Symbol | Tolerances |
| Mechanical Assembly |  |

File E132002	Vol. 13	Sec. 3	Page 4A	Issued:  2005-11-10

and Report	Revised:  2010-06-03

# ILLUSTRATIONS:

ILL. 1 – Top Side Of Main Board Trace Layout for Models AVO100-48S3V3

and ALO30F48N.

ILL. 2 – Inner Main Board Layout for Model AVO100-48S3V3.

ILL. 3 - Bottom Side Of Main Board Layout for Model AVO100-48S3V3

ILL. 4 – Top Side Of Main Board Trace Layout for Models AVO75-48S12

and AVO50-48S12.

ILL. 5 – Inner Main Board Layout for Models AVO75-48S12 and AVO50-48S12.

ILL. 6 - Bottom Side Of Main Board Layout for Models AVO75-48S12

and AVO50-48S12.

ILL. 7 – Top Side Of Main Board Trace Layout for Models AVO75-48S2V5

and AVO50-48S2V5.

ILL. 8 – Inner Main Board Layout for Models AVO75-48S2V5

and AVO50-48S2V5.

ILL. 9 - Bottom Side Of Main Board Layout for Models AVO75-48S2V5

and AVO50-48S2V5.

ILL. 10 – Top Side Of Main Board Trace Layout for Models AVO50-48S05

and AVO75-48S05.

ILL. 11 – Inner Main Board Layout for Models AVO50-48S05

and AVO75-48S05.

ILL. 12 - Bottom Side Of Main Board Layout for Models AVO50-48S05

and AVO75-48S05.

ILL. 13 – Top Side Of Main Board Trace Layout for Models AVO75-48S1V8

and AVO50-48S1V8.

ILL. 14 – Inner Main Board Layout for Models AVO75-48S1V8

and AVO50-48S1V8.

ILL. 15 - Bottom Side Of Main Board Layout for Models AVO75-48S1V8

and AVO50-48S1V8.

ILL. 16 – Top Side Of Main Board Trace Layout for Models AVO75-48S1V2

and AVO50-48S1V2.

ILL. 17 – Inner Main Board Layout for Models AVO75-48S1V2

and AVO50-48S1V2.

ILL. 18 - Bottom Side Of Main Board Layout for Models AVO75-48S1V2

and AVO50-48S1V2.

ILL. 19 – Top Side Of Main Board Trace Layout for Models AVO75B-48S3V3

and AVO50B-48S3V3.

ILL. 20 – Inner Main Board Layout for Models AVO75B-48S3V3

and AVO50B-48S3V3.

ILL. 21 - Bottom Side Of Main Board Layout for Models AVO75B-48S3V3

and AVO50B-48S3V3.

ILL. 22 – Main Board trace Layout for model AVO100B-48S3V3.

File E132002	Vol. 13	Sec. 3	Page 4B	Issued:  2005-11-10

and Report	Revised:  2010-06-03

MODEL DIFFERENCE:

# Model AVO100-48S3V3 is identical to Model ALO30F48N except for the Model Designation.

# Models AVO75-48S12 and AVO50-48S12 are identical to Model AVO100-48S3V3 except for the Power Transformer (T1), electric rating and PCB layout.

# Models AVO75-48S2V5 and AVO50-48S2V5 are identical to Model AVO100-48S3V3 except for the electric rating and PCB layout and alternate of Inductor (L1) and Current Transformer (CT1).

# Model AVO50-48S12 is identical to Model AVO75-48S12 except for

# electric rating.

# Model AVO50-48S2V5 is identical to Model AVO75-48S2V5 except for

# electric rating.

# Model AVO50B-48S3V3 is identical to Model AVO75B-48S3V3 except for

# electric rating.

# Model AVO75B-48S3V3 is identical to Model AVO100-48S3V3 except for the electric rating and PCB layout and alternate of Inductors (L1, L2) and Current Transformer.

# Models AVO75-48S05 and AVO50-48S05 are identical to Model AVO100-48S3V3 except for the power transformer (T1), electric rating and PCB layout.

# Model AVO50-48S05 is identical to Model AVO75-48S05 except for

# electric rating.

# Models AVO75-48S1V8 and AVO50-48S1V8 are identical to Model       AVO100-48S3V3 except for the electric rating and PCB layout and alternate of Inductors (L1, L2) and Current Transformer (CT1).

# Models AVO75-48S1V2 and AVO50-48S1V2 are identical to Model       AVO100-48S3V3 except for the electric rating and PCB layout and alternate of Inductors (L1, L2) and Current Transformer (CT1).

# Model AVO50-48S1V8 is identical to Model AVO75-48S1V8 except for

# electric rating.

# Model AVO50-48S1V2 is identical to Model AVO75-48S1V2 except for

# electric rating.

# Model AVO100B-48S3V3 is identical to Model AVO100-48S3V3 except for PCB

# layout and Inductors (L1, L2) and Current Transformer (CT1).

# GENERAL:

General - The general design, shape and arrangement shall be as illustrated, in the following figures, except where variations are specifically described.

File E132002	Vol. 13	Sec. 3	Page 5	Issued:  2005-11-10

and Report	Revised:  2012-12-07

# Model AVO100-48S3V3 – FIG.1

General – Fig. 1 shows the Top view of Model AVO100-48S3V3.  Also represents Models ALO30F48N, AVO75-48S12, AVO50-48S12, AVO75-48S2V5,

AVO50-48S2V5, AVO75-48S05,  AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2,   AVO50-48S1V8, AVO50-48S1V2, AVO75B-48S3V3,  AVO50B-48S3V3 and AVO100B-48S3V3.

1.	Printed Wiring Board - See Section General for details.  Measured 57.9 by 22.8 mm, minimum 3.0 mm thick.  Rated min. V-1.  See ILL. 1 for trace layout for Models AVO100-48S3V3 and ALO30F48N, See ILL. 4 for Models AVO75-48S12 and AVO50-48S12, See ILL.7 for Models AVO75-48S2V5 and AVO50-48S2V5, See ILL.10 for Models AVO75-48S05 and AVO50-48S05, See  ILL. 13 for Models AVO75-48S1V8 and AVO50-48S1V8, See ILL. 16 for Models AVO75-48S1V2 and AVO50-48S1V2, See ILL.19 to 21 for                 Models AVO75B-48S3V3 and AVO50B-48S3V3, See ILL. 22 for model AVO100B-48S3V3.

2.	Power Transformer (T1) - Windings etched in main PWB.  Core: overall 19.8mm by 6.6 mm by 8.0mm.  See ILL. 2 for details of AVO100-48S3V3 and ALO30F48N, See ILL. 5 for details of AVO75-48S12 and AVO50-48S12, See ILL. 8 for details of AVO75-48S2V5 and AVO50-48S2V5, See ILL.11 for Models AVO75-48S05 and AVO50-48S05, See ILL. 14 for Models AVO75-48S1V8 and AVO50-48S1V8, See ILL. 17 for Models AVO75-48S1V2 and AVO50-48S1V2.

For model AVO100B-48S3V3, core: overall 18.6mm by 8.2mm by 8.0mm, See ILL. 22 for details.

3.	Current transformers (CT1) – For Models AVO100-48S3V3, ALO30F48N,

AVO75-48S12, AVO50-48S12, AVO75-48S05 and AVO50-48S05.  Emerson Network Power, Type AQD1A331541CT1.  For Models AVO75-48S2V5 ,AV050-48S2V5, AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2 and AVO100B-48S3V3, Emerson Network Power, Type AQS1K3341CT1.

4.	Current Transformer Bobbin – R/C, (QMFZ2), Sumitomo Bakelite Co. Ltd. (E41429), Type PM-9820, rated V-0, 150°C.

Alternate – Same as above except Type PM-9630, rated V-0, 150°C.

Alternate - Same as above except Sumitomo Chemical Co. Ltd (E54705), Type E4008, V-0, 130 °C.

Alternate - Same as above except Sumitomo Chemical Co. Ltd (E54705), Type E4008L, V-0, 130 °C.

5.	Inductor (L1) -

For Models AVO100-48S3V3, ALO30F48N, AVO75-48S12, AVO50-48S12,       AVO75-48S05 and AVO50-48S05, Emerson Network Power, Type AOS1A3341L1, rated 130°C.

For Models AVO75-48S1V8, AVO75-48S1V2, AVO50-48S1V8, AVO50-48S1V2, AVO75-48S2V5, AVO50-48S2V5, AVO75B-48S3V3, AVO50B-48S3V3 and AVO100B-48S3V3, Emerson Network Power, Type AOS753341L1, rated 130°C.

Alternate – For Model AVO100B-48S3V3, same as above except for Type AOS1A3345BL1.

File E132002	Vol. 13	Sec. 3	Page 5A	Issued:  2005-11-10

and Report	Revised:  2010-06-03

6.	Inductor (L2) - For Models AVO100-48S3V3, ALO30F48N, AVO75-48S12,

AVO50-48S12, AVO75-48S05, AVO50-48S05, AVO75-48S2V5 and AV050-48S2V5,

Emerson Network Power, Type AQS753341L3, rated 130°C.  For Models

AVO75-48S1V8 and  AVO50-48S1V8, Emerson Network Power, Type AOS751841L2,

rated 130°C.  For Models AVO50-48S1V2 and  AVO75-48S1V2, Emerson Network

Power AQD75331541L1, rated 130°C.  For Models AVO75B-48S3V3,

AVO50B-48S3V3, and AVO100B-48S3V3, Emerson Network Power, Type

AST65341L3, rated 130°C.

7.	Inductor (L3) - Winding etched in main PWB.  Core: overall measures 19.8mm by 6.6 mm by 8.0mm.  See ILL. 2 for details of AVO100-48S3V3 and ALO30F48N, See ILL. 5 for details of AVO75-48S12 and AVO50-48S12, See ILL. 8 for details of AVO75-48S2V5 and AVO50-48S2V5. See ILL.11 for details of AVO75-48S05 and AVO50-48S05, See ILL. 14 for Models AVO75-48S1V8 and AVO50-48S1V8, See ILL. 17 for Models AVO75-48S1V2 and AVO50-48S1V2, See ILL. 19 to 21 for Models AVO75B-48S3V3 and AVO50B-48S3V3.

For model AVO100B-48S3V3, core: overall measures 18.6mm by 8.2mm by 8.0mm, See ILL. 22 for details.

8.	Heat sinks  – Overall measured 3.0 by 2.5 by 3.0mm.  For Models AVO100-48S3V3, ALO30F48N, AVO75-48S12, AVO50-48S12, AVO75-48S05, AVO50-48S05, AVO75-48S2V5 and AV050-48S2V5, Three provided.  For Models AVO75-48S1V8, AVO50-48S1V8, AVO75-48S1V2 and AVO50-48S1V2, Two provided,.

9.	Bridging capacitors (C6, C6A) – Two provided.  Each rated 2 KV minimum, 1000 pF maximum.

10.	Power Transformer (only for T2 of Models AVO75B-48S3V3 and 	                AVO50B-48S3V3) - Windings etched in main PWB.  Core: overall 19.8 mm by 6.6 mm by    8.0 mm.  See ILL. 19 to 21 for details.

11.	Current transformer (only for T1 of Models AVO75B-48S3V3 and      AVO50B-48S3V3), Emerson Network Power, Type AQS1K3341CT1.

12.	Current Transformer Bobbin (only for T1 of Models AVO75B-48S3V3 and      AVO50B-48S3V3) – R/C, (QMFZ2), Sumitomo Bakelite Co. Ltd. (E41429), Type PM-9820, rated V-0, 150°C.

Alternate – Same as above except Type PM-9630, rated V-0, 150°C.

13.	Bridging capacitor (only for C6 of Models AVO75B-48S3V3 and       AVO50B-48S3V3) – Murata Munufacturing Co LTD, Type X7R, rated maximum 1500 pF, rated 125 °C with isolation test voltage minimum 1.5 KV.

Alternate – Same as above except for Johanson Dielectrics Inc (E212609), Type 302R29 series.

Alternate – Maximum 1500 pF, with isolation test voltage minimum 1.5 KV. See Section General for details.

14.   Heat sink case (for model AVO100B-48S3V3) – Optional, metal, overall

57.8mm by 23.0mm. Mounted on PWB by solder.

File E132002	Vol. 13	Sec. 3	Page 6	Issued:  2005-11-10

and Report	Revised:  2012-12-07

Model AVO100-48S3V3 – FIG. 2

General – Fig. 2 shows the Bottom View of Model AVO100-48S3V3.  Also represents Models ALO30F48N, AVO75-48S12, AVO50-48S12, AVO75-48S2V5,

AVO50-48S2V5, AVO75-48S05, AVO50-48S05, AVO75-48S1V8, AVO75-48S1V2,       AVO50-48S1V8, AVO50-48S1V2, AVO75B-48S3V3, AVO50B-48S3V3 and AVO100B-48S3V3..

*1.	Optocouplers (OPT01, OPT02) – Two provided.  (FPQU2), NEC Compound Semiconductor Devices Ltd. (E72422), Type PS2801-1, rated 2500 V ac isolation test voltage.

Alternate – R/C, (FPQU2), rated min. 2000 V dc. See Section General for details.